# Manual Do it yourself Chameleon Scope

## Required material

To create the R2D part of the Chameleon Scope you need the following parts:

- Camera module ESP32-CAM

- 400mAh LiPo battery 3.7V

- 3D printer and filament (TPU and PLA) for the case

- Battery 5V booster module

- Camera extension

- Cable for soldering

- I/O switch

## Soldering

The ESP32-CAM requires 5V, while the LiPo battery transfers 3.7V voltage. To transform the voltage, it needs a booster module in between. One advantage of the booster module is that it has an integrated USB-C interface, which allows the ESP32 to be charged via the booster module. With the help of this circuit diagram, the soldering is carried out and explained step by step.

Step 1: Soldering the battery to the booster module

|  | - Start with the battery and solder its positive terminal to the B+ surface. - Solder the negative terminal of the battery to the B- surface. - Take the black cable and solder it to the outermost pin on the B side (blue marking). - Take the red cable and solder it to the outermost pin on the B+ side (red marking). |
| --- | --- |

Step 2: Soldering the cables coming from the booster module to the switch

|  | - Begin at the booster module. - Solder the power supply (red cable) to the outer pin of the switch. - Solder another power supply (red cable) to the middle pin of the switch. - Connect the cable from the middle pin of the switch to the 5V pin of the microcontroller. - From the booster module, solder the earth (black cable) directly to the GND pin of the microcontroller, located below the 5V pin. - This creates a circuit that can be interrupted by the switch. |
| --- | --- |

## Runtime

Runtime Microcontroller

Battery: 400 mAh LiPo Battery– 3,7 V

Booster Modul: 5V

Battery power

400 mAh * 5V

= 0,4 *5

= 2 Wh (2 Watt per hour)

Active Camera

P = 140 mW

P = 0,14 W

Runtime = 2 Wh / 0,14 W

Runtime = 14 Hours

Active flashlight

i = 0,31 A

P = 5V * 0,31 A

P = 1,55 W

Runtime = 2 Wh / 1,55 W

Runtime = 1,29 h = 1 hour 18 minutes

## IT-Architecture

## 3D-Case explanation

Inner Case:

- Variables: Dimensions and translations defined.

- Code structure: Uses difference() and cube() functions.

- Case: Outer case created with cube() function.

- Inside: Interior space created with cube() function.

- Camera: Cylindrical camera object created with cylinder() function.

- LED: Cylindrical LED object created with cylinder() function.

- Cap: Cap created with cube() function.

- Gap to open: Gap for opening case created with cube() function.

- Magnet: Magnet created with cube() function.

- Holding to open: Holding structures and gap holding objects created with cube() function.

Outside-Case:

- Variables: Case dimensions and translations defined.

- Code structure: Uses difference() and minkowski() functions.

- Camera: Camera object created with cylinder() function.

- Case: Outer case created with cube() function.

- Additional Components: Spheres and fixation objects created with minkowski(), cylinder(), and cube() functions.

- Fixation: Fixation objects created with cylinder() and cube() functions.

Cap:

- Translation and Cube: Cube created with cube() function.

- Plattform: Cylindrical and cubic objects created with cylinder() and cube() functions.

- Fixation: Cylindrical and cubic objects created with cylinder() and cube() functions